# Tool/Equipment Details

Tool/Equipment Description. Brand; Model / Identification No.; Sydney Trains Asset No.. IR Tester Asset No.:; IR Tester Next Calibration Date:.

# For guidance on how to complete this form, please refer to:

SP D 79041 Insulated Sticks, Tools and Equipment used for work near or on/within Exposed Electrical Equipment – Inspection, Testing, Care and Maintenance D2013/80872 Inspection and Testing of Insulated Sticks, Tools and Equipment used for work On or Near Exposed 1500V OHW or Equipment

# The Insulated Stick, Tool or Equipment is used for work On or Near Exposed 1500V?

Delete the appropriate words from the heading above. In documenting this determination refer to the work specific SWI and D2013/80872 section titled “Inspection and Test Philosophy”. Note: The IR tests to be performed are dictated by this determination.

# Visual Inspection

Check finish of insulated stick, tool, equipment and accessories has no cracks, surface damage, or mechanical defects, etc. A 3 month inspection period is only required for tools & equipment used “On” exposed1500V; otherwise it’s an annual inspection. For more details, please refer to SP D 79041 Section 10.

# Insulation Resistance (IR) Test

The IR test is conducted on the tool or equipment, section by section or as a complete assembly, in either a Dry or Wet condition. For more details and guidance refer to D2013/80872 section titled “IR Test Procedure”.

| Date | Enter if Inspection or Wet/Dry IR test was conducted | Record IR value (MΩ) (Pass > 50 MΩ Fail < 50 MΩ) | Comments (e.g. condition of insulating surface) | Result (Pass or Fail) | Tested by (Name & Signature) |
| --- | --- | --- | --- | --- | --- |